Abstract

There's an increasing interest from customers in new, healthier snack choices, so we need to look into making them with local grains and root crops. This research focuses on creating and refining Rizo Crisp, a snack made from rice and potato. It has cumin, salt, and chili powder for flavor. The goal was to find the right mix of ingredients and cooking steps to get a snack that tastes good, is nutritious, and has good qualities. Three different recipes (T1, T2, T3) were made, each with different amounts of rice, potato, and seasonings. The best recipe (T2: 60% rice, 30% potato) was cooked by extrusion and then deep-fried. Lab tests showed that 100g of the final product has 517 calories, 5.4g of protein, 29.2g of fat, 56.1g of carbs, and 2g of fiber. Taste tests showed that T2 had the best ratings for texture, taste, and overall liking. In short, Rizo Crisp—especially the T2 version—is a successful new snack that could do well commercially since it uses rice and potato in a good way.

Keywords: Extruded Snack, Rice, Potato, Sensory Evaluation, Optimization, Proximate Composition

Introduction

The world snack food market is continuously evolving, with a rising demand for novel, convenient, and healthier products. Extruded puffed snacks, such as corn puffs and rice crisps, remain highly popular due to their light texture, crispiness, and convenience . Starch-based materials, particularly from cereals and tubers, are the primary ingredients for such products, with their functional properties dictating the final product quality. Rice is a common base for extruded snacks due to its excellent expansion properties, while potato contributes to binding and can enhance crispness . The physicochemical transformations of starch during extrusion, such as gelatinization and degradation, are well-documented and critically influence texture and struc Honestly,ture . Recent research has also explored the impact of specific process parameters, like screw speed and barrel temperature, on the attributes of extruded snacks . The originality of this work lies in the development of a hybrid starch system combining rice and potato flours in a "Kurkure"-type geometry. Honestly, while both ingredients have been studied individually, their combination in an extruded, spiral-shaped snack infused with spices like cumin and chilli prior to frying is not widely reported. This study systematically investigates the effect of blend ratio and processing conditions on the expansion, oil uptake, texture, and sensory properties of the novel Rizo Crisp, thereby contributing to the field of composite starch snack development.

Material and Methods

Raw Materials

Commercial-grade rice flour, fresh potatoes (boiled, mashed, and dried), powdered cumin seeds, red chilli powder, salt, and sunflower oil were procured for the study.

Experimental Design and Formulation

We developed three initial formulations (T1, T2, T3) with varying ingredient proportions, as detailed in Table 1. Based on preliminary trials, formulation T2 was selected as the optimized final product for detailed analysis (Table 2).

Production Process

The production process involved the following steps:

1) Weighing and pre-treatment of raw materials;

2) Blending of rice flour, potato mash, and spices (cumin, chilli, salt);

3) Hydration and mixing to achieve a uniform dough;

4) Shaping using a screw extruder fitted with a die to form the characteristic curly shapes;

5) Deep-frying in a controlled fryer at 165°C until the moisture content reduced to approximately 1.5-2%;

6) De-oiling and cooling to ambient temperature; and

7) Packaging in food-grade LDPE pouches.

Physicochemical and Nutritional Analysis

We analyzed the optimized product (T2) for its proximate composition. Moisture, ash, fat, protein, and carbohydrate content were determined using standard laboratory methods. The energy values are calculated . Furthermore, physicochemical properties such as expansion ratio and oil uptake were measured.

Sensory Evaluation

The three formulations (T1, T2, T3) were evaluated sensory by a panel of untrained judges. Attributes including colour & appearance, texture, taste, flavour, and overall acceptability were scored using a 9-point hedonic scale (1 = dislike  , 9 = like ).

Statistical Analysis

Sensory data were analyzed using one-way Analysis of Variance (ANOVA). Post-hoc tests were conducted to identify significant differences between treatment means where applicable (p<0.05).

Results and Discussion

Proximate Composition

The proximate composition of the optimized Rizo Crisp (T2) is presented in Table 3. The product provided 517 kcal per 100g, with 5.4g protein, 29.2g fat, and 56.1g carbohydrates. The dietary fiber content was 2g, and ash content was 4.3g. The fat content is attributed to the deep-frying process, which is characteristic of this snack category. The formulation successfully created a palatable snack with a balance of macronutrients.

Sensory Evaluation

The mean sensory scores for all treatments are summarized in Table 4. Statistical analysis (Table 5) revealed significant differences (p<0.05) among the treatments for all attributes.

Treatment T2 consistently received the highest scores for texture (8), taste (7.5), flavour (7.5), and overall acceptability (7). The balanced rice-to-potato ratio (60:30) in T2 likely contributed to its superior texture, providing optimal expansion and crispness without excessive hardness or oiliness. In contrast, T3, with a higher potato content, received the lowest scores for texture and overall acceptability, suggesting that excessive potato can lead to a denser, less appealing product. T1, while acceptable, scored lower on flavour, indicating that the seasoning level in T2 was more optimal.

These findings are consistent with literature on composite snacks, where the ratio of ingredients significantly impacts sensory perception and texture. The high overall acceptability of T2 demonstrates that the optimized formulation successfully balances the functional properties of both starches with the sensory appeal of the spice blend.

Process Optimization

Observations during processing indicated that a moisture content of 14% and a frying temperature of 165°C were optimal for the T2 formulation. At this combination, expansion was highest and oil uptake was moderate. This aligns with studies on extruded snacks, where moisture content and thermal energy input are critical determinants of expansion and final product quality.

Conclusion

Our study successfully developed a novel extruded snack, Rizo Crisp, from a blend of rice and potato flour. The optimized formulation (T2) demonstrated a desirable nutritional profile and was the most sensorially accepted variant. The hybrid starch system proved effective in creating a product with excellent crispness and expansion. The integration of spices directly into the dough ensured uniform flavour distribution. This product showcases significant potential as a commercial snack, offering a new option in the competitive extruded snack market. Future work could focus on shelf-life stability, alternative cooking methods like baking to reduce fat content, and detailed microstructural analysis.

Acknowledgement

We express our sincere gratitude to the faculty and staff of the Department of Food Technology for their support and guidance throughout this research project. We also thank the sensory panelists for their participation and valuable feedback.

References

1.  Anchondo-Trejo A., et al., Development of a Third Generation Snack of Rice Starch, Molecules, 26(1), 54 (2021)

2.  Xu J., et al., Functional Food Based on Potato, Foods, 12(11), 2145 (2023)

3.  Lisiecka K., et al., Structure and Texture Characteristics of Novel Snacks, Materials, 16(4), 1541 (2023)

4.  Qiu J., et al., Research Progress on the Physicochemical Properties of Starch‐Based Foods by Extrusion Processing, Foods, 13(22), 3677 (2024)

5.  Jacquet A., et al., Impact of Operating Parameters and Fat Addition on Texture and Physicochemical Attributes of Extruded Snacks , Foods , 14 (8), 1307 (2025)

Declaration by Authors

The facts and views in the manuscript are ours and we are responsible for authenticity, validity and originality etc. We undertake and agree that the manuscript submitted to your journal has not been published elsewhere and has not been simultaneously submitted to other journals. We also declare that the manuscript is our original work and we have not copied it from anywhere else. There is no plagiarism in our manuscript. Our manuscript, whether accepted or rejected, shall be the property of the publisher of the journal and all the copyrights will be with the publisher of the journal.

Separate Sheets for Tables and Figures

Table 1: Initial formulations for Rizo Crisp (g/100g batch).

| Parameter | T1 | T2 | T3 |
| --- | --- | --- | --- |
| Ingredient |  |  |  |
| Rice flour | 90 | 60 | 50 |
| Potato | - | 30 | 40 |
| Cumin seeds | 6 | 4 | 5 |
| Red chilli powder | 2 | 2 | 2.5 |
| Salt | 2 | 4 | 2.5 |

Table 2: Optimized final formulation (T2) for Rizo Crisp (g/100g batch).

| Parameter | T2 |
| --- | --- |
| Ingredient |  |
| Rice flour | 60 |
| Potato | 30 |
| Cumin seeds | 4 |
| Red chilli powder | 2 |
| Salt | 4 |

Table 3:Proximate composition of Rizo Crisp (T2) per 100g.

| Chemical Parameters | RIZO CRISP |
| --- | --- |
| Energy | 517 kcal |
| Carbohydrate | 56.1g |
| Fat | 29.2g |
| Protein | 5.4g |
| Ash | 4.3g |
| Fiber | 2g |
| Moisture | 3g |

Table 4: Mean sensory scores of different Rizo Crisp formulations.

| Sample | Colour and appearance | Texture | Taste | Flavour | Overall Acceptability |
| --- | --- | --- | --- | --- | --- |
| TO(Control Sample) | 8 | 8 | 8 | 8 | 8 |
| T1 | 6 | 7 | 7 | 6 | 6.5 |
| T2 | 7 | 8 | 7.5 | 7.5 | 7 |
| T3 | 8 | 5 | 6 | 6 | 5 |

Table 5: Statistical analysis (ANOVA) of sensory data.

| Attribute | Treatment Means |  |  | F-value | p-value | Significant Differences |
| --- | --- | --- | --- | --- | --- | --- |
|  | T1 | T2 | T3 |  |  |  |
| Colour | 6 | 7 | 8 | 217.67 | <0.0001 | T3 > T2 > T1 |
| Texture | 7 | 8 | 5 | 450.00 | <0.0001 | T2 > T1 > T3 |
| Taste | 7 | 7.50 | 6 | 112.50 | <0.0001 | T2 > T1 > T3 |
| Flavour | 6 | 7.50 | 6 | 187.50 | <0.0001 | T2 > T1 = T3 |
| Overall Acceptability | 6.50 | 7 | 5 | 112.00 | <0.0001 | T2 > T1 > T3 |

Figure 1: Flow process chart for the preparation of Rizo Crisp.

Raw Materials

Weighing & Pre-treatment

Blending

Hydration

Mixing with Potato Mash

Shaping

Thermal Processing

De-oiling

Cooling

Packaging

Quality

Data Collection & Optimization

Final Product